Great idea. Let me lay out the full content here, structured and thorough. We'll turn it into the interactive HTML afterwards.

# Resonant Converters — From First Principles

## 0. Where We Start From

You already understand ZVS in a half-bridge under inductive load. The key insight you have is this: if the inductor current is still flowing in its original direction when a MOSFET is about to turn on, the body diode of that MOSFET is already conducting, the switch node is clamped, and the MOSFET turns on with zero volts across it. No capacitive energy dump. No hard switching loss.

Resonant converters engineer that condition deliberately and reliably on every single switching cycle, across a wide load range. Everything we cover below is just a consequence of that one goal.

## 1. The LC Tank Circuit — Foundation of Everything

### 1.1 What is an LC Tank?

An inductor L and capacitor C connected together. Charge the capacitor to some voltage V₀, close the circuit, and energy begins oscillating between the electric field of the capacitor and the magnetic field of the inductor — forever, in an ideal lossless world.

### 1.2 Why the Current is Always Sinusoidal

This is the single most important conceptual point to internalise. Let's derive it properly.

Apply KVL around a series LC loop with no source and no resistance:

V_L + V_C = 0

L · di/dt + (1/C)·∫i dt = 0

Differentiate with respect to time:

L · d²i/dt² + i/C = 0

→  d²i/dt² + (1/LC) · i = 0

This is the simple harmonic oscillator equation. Its general solution is:

i(t) = I_peak · sin(ω₀t + φ)

where  ω₀ = 1/√(LC)   [rad/s]

f₀  = 1/(2π√(LC))   [Hz]   ← resonant frequency

There is no other solution. The differential equation forces sinusoidal behaviour. This is identical in form to a pendulum or a mass-spring system:

- The inductor provides inertia — it resists change in current, exactly like mass resists change in velocity

- The capacitor provides the restoring force — like a spring, it pushes back proportionally to displacement (charge)

Energy oscillates: magnetic field (kinetic) ↔ electric field (potential). The peak current is set by the characteristic impedance:

Z₀ = √(L/C)    [ohms]

I_peak = V_initial / Z₀

A high Z₀ means the tank is "stiff" — a large voltage only drives a small current. A low Z₀ means it is "soft" — the same voltage drives a large current. This is crucial for sizing (covered in Section 8).

### 1.3 The 90° Phase Relationship

In a free-running LC tank:

- Capacitor voltage: cosine (starts at maximum)

- Inductor current: sine (starts at zero)

They are always 90° apart. This is not a coincidence — it is a direct consequence of the energy exchange. Maximum voltage = all energy in capacitor = zero current. Zero voltage = all energy in inductor = maximum current.

## 2. Driving the Tank from a Bridge — Impedance and Phase

### 2.1 What Happens When We Drive with a Square Wave

When a half-bridge or full-bridge drives the LC tank, it applies a square wave at switching frequency f_sw. The tank doesn't just oscillate freely anymore — it responds to the driving source.

The series impedance of an LC network at angular frequency ω is:

Z_LC(jω) = jωL + 1/(jωC)

= j(ωL − 1/ωC)

This is purely reactive (imaginary). The magnitude is:

|Z_LC| = |ωL − 1/ωC|

At resonance: ωL = 1/ωC, so |Z_LC| = 0. The tank is a short circuit at its resonant frequency. Maximum current flows.

### 2.2 The Two Operating Regions

This is where ZVS lives or dies.

| Condition | f_sw vs f₀ | Net reactance | Current vs Voltage | Name |
| --- | --- | --- | --- | --- |
| f_sw < f₀ | Below resonance | Capacitive (1/ωC dominates) | Current leads voltage | Capacitive region |
| f_sw = f₀ | At resonance | Zero | In phase | Resonance point |
| f_sw > f₀ | Above resonance | Inductive (ωL dominates) | Current lags voltage | Inductive region ✓ |

The inductive region is where we want to be, always. Current lagging means current is still flowing in its previous direction at every switching instant. That means the body diode is conducting. That means ZVS.

### 2.3 Why Capacitive Region Kills ZVS

In the capacitive region, current leads voltage. This means the current has already reversed by the time the switching edge arrives. The body diode is OFF. The MOSFET turns on with full bus voltage across it. The energy stored in C_oss — which is ½·C_oss·V_bus² — gets dumped as heat through the channel in a nanosecond-scale current spike. That is hard switching at turn-on.

Worse: if the body diode was conducting in the capacitive region (which it briefly will be), it gets hit with reverse recovery when the current reverses. Body diodes are slow — they were not designed to be rectifiers. Reverse recovery injects a spike of reverse current and then abruptly stops, creating a current spike and EMI. This is the other reason we never want to operate below resonance in a resonant converter.

### 2.4 The Phase Angle and Its Physical Meaning

For a series RLC at driving frequency f_sw (including a reflected load resistance R_ac):

φ = arctan((ωL − 1/ωC) / R_ac)

Where φ is the angle by which current lags voltage.

- φ > 0 → inductive → ZVS possible ✓

- φ < 0 → capacitive → ZVS fails ✗

- φ = 0 → at resonance → marginal (need magnetising current)

At the switching instant, the current value is:

I_at_switching_instant = I_peak · sin(φ)

This current must be large enough to swing both MOSFETs' C_oss across the full bus voltage during the dead time. More phase = more current at the switching instant = better ZVS margin. But more phase also means more reactive (circulating) current for the same real power delivered — which means more conduction loss. This is the fundamental trade-off of resonant converters. You buy ZVS with reactive current.

## 3. ZVS — The Full Commutation Sequence

Let's trace exactly what happens in a half-bridge during one transition, with the tank in the inductive region. This is the same mechanism you know — we are just being precise about it.

### 3.1 Energy Stored in C_oss

Every MOSFET has output capacitance C_oss. During a dead-time interval, the switch node must swing from one rail to the other. Both MOSFETs' C_oss values appear in parallel from the switch node's perspective:

C_oss_eff = C_oss_Q1 + C_oss_Q2   (during dead time)

Q_required = C_oss_eff × V_bus     (charge to move)

E_Coss = ½ · C_oss_eff · V_bus²   (energy to move)

The inductor current must supply this charge during the dead time.

Critical C_oss note: C_oss is highly nonlinear. It is very large near 0V (can be 5–10× the datasheet nominal value) and drops sharply as V_DS rises. For dead time calculations, always use Q_oss from the datasheet (the charge vs voltage curve integrated to V_bus), not a simple C × V. Using a single C_oss value will give you wrong answers.

### 3.2 Step-by-Step Commutation (Inductive Region, Low-side Turn-on)

Phase 1 — Q1 conducting:
Tank current is positive (flowing into the switch node from above). Q1 channel carries the current. Switch node is at +V_bus.

Phase 2 — Q1 turns OFF, dead time begins:
Q1 gate goes low. Channel turns off. But the inductor current cannot stop — Lenz's law. It must continue flowing. It now flows through:

- Q1's C_oss — charging it from 0V toward V_bus

- Q2's C_oss — discharging it from V_bus toward 0V

The switch node voltage begins to fall at a rate determined by:

dV_sw/dt = I_L / C_oss_eff

Phase 3 — Switch node reaches 0V:
If the inductor current is large enough (Q_required = C_oss_eff × V_bus is supplied within the dead time), the switch node reaches 0V and Q2's body diode clamps it. V_DS across Q2 is now approximately 0.7V (body diode drop).

Phase 4 — Q2 gate turns ON: ZVS achieved ✓
The gate signal arrives. Since V_DS ≈ 0, the MOSFET turns on with zero voltage across it. No C_oss energy dump. No turn-on switching loss. The MOSFET channel takes over from the body diode.

Phase 5 — Current reversal:
With Q2 ON and the switch node at 0V, the tank sees the reversed driving voltage. Current ramps through zero and goes negative. This prepares the turn-on of Q1 in the next half cycle.

### 3.3 What If the Current is Too Small?

If I_L at the switching instant is insufficient to complete the C_oss swing, the switch node does not reach 0V before the gate arrives. The MOSFET turns on with residual voltage across it — partial hard switching. The remaining capacitive energy ½ C_oss (V_remaining)² is dissipated at turn-on. This is why ZVS coverage at light load is a key design challenge.

## 4. Dead Time — The ZVS Timing Window

Dead time is the gap between turning one switch OFF and the other ON. It is the window in which the C_oss swing must complete.

### 4.1 Minimum Dead Time

t_dead_min = Q_oss(V_bus) / I_L_at_transition

≈ C_oss_eff × V_bus / I_L   (simplified, assuming constant current)

Practical rule:

t_dead_target ≈ 1.5 × t_dead_min

This gives margin for:

- C_oss nonlinearity (slower swing near 0V where C_oss is large)

- Layout inductance adding delay

- Current variation across load range

### 4.2 Maximum Dead Time

Too long is also bad:

- Body diode conduction loss (body diodes have ~0.7V drop, much worse than MOSFET RDS_on)

- In LLC, excessively long dead time allows the magnetising current to reverse direction during dead time, losing ZVS

- In general, dead time is wasted time — it reduces effective duty cycle

t_dead_max ≪ T_sw/2 = 1/(2·f_sw)

### 4.3 Phase Angle, Dead Time, and the LLC Lm Constraint

Here is where dead time connects to component sizing in a beautiful way. In the LLC:

At light load, the series tank current (which carries the real power) drops toward zero. The only current available for the C_oss swing is the magnetising current I_Lm — which always flows regardless of load because L_m is always magnetised by the applied voltage.

I_Lm_peak = V_in / (4 · f_sw · L_m)   [half-bridge, approximate]

ZVS at light load requires:

½ · L_m · I_Lm_peak² ≥ ½ · C_oss_eff · V_bus²

Substituting:

L_m ≤ V_bus² · t_dead / (C_oss_eff · V_bus²)  →  L_m ≤ t_dead / C_oss_eff

[simplified form]

More usefully:

I_Lm_peak ≥ C_oss_eff · V_bus / t_dead

→  L_m ≤ t_dead · V_in / (4 · f_sw · C_oss_eff · V_bus)

This gives you a hard upper bound on L_m. You cannot make L_m arbitrarily large to reduce magnetising current (and therefore reduce no-load losses), because you need that current for ZVS. The C_oss value, the dead time, and the bus voltage jointly constrain L_m. This is the direct answer to your friend's "calculate C_oss and hope" comment — it is not hope, it is a hard constraint that directly sizes L_m.

## 5. The Dual Active Bridge (DAB)

### 5.1 Topology

Two full H-bridges linked by a transformer. The transformer's leakage inductance (or an added series inductance) is the only reactive element between the two bridges. No resonant capacitor in the basic DAB.

[H-Bridge 1] ---L_series--- [Transformer n:1] --- [H-Bridge 2]

V_in                                               V_out

Both bridges switch at the same frequency f_sw. The angle between their switching patterns — the phase shift φ — controls power flow magnitude and direction.

### 5.2 Why the Current is Trapezoidal (Not Sinusoidal)

This is a key conceptual distinction. In an LC tank, there is a capacitor — so the governing equation is second-order and forces sinusoidal current. In the DAB, there is only a single inductor between two voltage sources. The governing equation is:

V_L = V_primary(t) − n·V_secondary(t)

di/dt = V_L / L

Both V_primary and V_secondary are square waves (constant levels within each half-period). So V_L is also a piecewise-constant waveform, and therefore di/dt is piecewise constant — the current ramps linearly up or down in straight lines. This produces a trapezoidal (or triangular) current shape.

There is no resonance. The current shape is purely determined by volt-second balance on the inductor. This is the fundamental difference between the DAB and the LLC.

### 5.3 Power Transfer Equation

P = (V_in · V_out_reflected · φ) / (2π · f_sw · L) · (1 − |φ|/π)

where φ = phase shift between bridges [radians]

Maximum power at φ = π/2 (90°)

Power reverses sign when φ reverses sign → bidirectional naturally

### 5.4 DAB ZVS Condition and Its Weakness

ZVS in the DAB requires the inductor current to be flowing in the correct direction at each switching instant. At full load and matched voltage ratio (V_in = n·V_out), the current waveform is nicely triangular and ZVS is achievable on all four switches.

However, at light load (small φ), the current amplitude is small. At the switching instant, there may not be enough current to swing C_oss — ZVS is lost. At mismatched voltage ratio (V_in ≠ n·V_out), the current waveform becomes asymmetric and ZVS is lost on some transitions.

This is the classic DAB weakness. Advanced modulation schemes (Extended Phase Shift EPS, Dual Phase Shift DPS, Triple Phase Shift TPS) add extra degrees of freedom in the gating to reshape the current and maintain ZVS over wider operating ranges — but they significantly complicate the control.

### 5.5 DAB With MOSFETs vs Diodes

The basic DAB uses MOSFETs on both sides — this is what makes it naturally bidirectional. Power flows from bridge 1 to bridge 2 for positive φ, and from bridge 2 to bridge 1 for negative φ. Both bridges must be active (not diode rectifiers) for this to work.

## 6. The LLC Resonant Converter

### 6.1 The Three-Element Tank

LLC adds a capacitor C_r in series with L_r, and uses the transformer's magnetising inductance L_m as the third element. Hence L (series) – L (magnetising) – C (series) = LLC.

[Half-Bridge] --- L_r --- C_r ---+--- [Transformer primary (L_m in parallel)] --- [Diode bridge] --- V_out

|

(L_m is physically the transformer's magnetising inductance,

not a separate component — though it can be)

The secondary is typically a diode bridge (not MOSFETs) for unidirectional operation. This is important and we will return to it.

### 6.2 Two Resonant Frequencies

Because there are three reactive elements, the LLC has two resonant frequencies:

f_r1 = 1/(2π√(L_r · C_r))                ← series resonance of L_r and C_r alone

f_r2 = 1/(2π√((L_r + L_m) · C_r))        ← lower resonance, when L_m participates

(at no load, since load clamps L_m)

Always: f_r2 < f_r1

The ratio k = L_m/L_r (inductance ratio) sets the separation between f_r1 and f_r2:

f_r2/f_r1 = 1/√(1 + 1/k) = √(k/(k+1))

Larger k → f_r2 closer to f_r1 → narrower operating frequency range.

### 6.3 Operating Regions and Gain Curves

The LLC voltage gain (V_out/V_in normalised) is analysed using the First Harmonic Approximation (FHA) — treating the tank input as the fundamental of the square wave. The gain as a function of normalised frequency f_n = f_sw/f_r1 and quality factor Q at full load:

Key parameters:

f_n = f_sw / f_r1    (normalised switching frequency)

k   = L_m / L_r      (inductance ratio, typically 3–8)

Q   = √(L_r/C_r) / R_ac    (quality factor at full load)

R_ac = (8n²/π²) · R_load   (reflected load resistance)

At f_sw = f_r1 (f_n = 1):

- Gain = 1 for all load conditions (Q values)

- C_r and L_r cancel each other (series resonance)

- The tank current is purely sinusoidal, in phase with the fundamental of the bridge voltage

- Maximum efficiency point — minimum current stress

At f_sw > f_r1 (f_n > 1 → inductive region, buck):

- Gain < 1

- The net tank reactance is inductive

- Current lags voltage → ZVS guaranteed

- Used when V_in is above nominal (need to buck down)

At f_r2 < f_sw < f_r1 (boost region):

- Gain > 1

- L_m starts participating in resonance with L_r and C_r

- ZVS still maintained in most of this region

- Used when V_in is below nominal (need to boost up)

At f_sw < f_r2:

- Gain behaviour changes — avoid this region

- Capacitive operation, ZVS lost, diode reverse recovery

Rule: Never operate below f_r2. Always stay between f_r2 and some maximum f_sw above f_r1. The LLC regulates V_out by varying f_sw within this window.

### 6.4 Why the Secondary Diodes Get ZCS for Free

Because the tank current is sinusoidal, it naturally passes through zero twice per cycle. The diodes on the secondary simply stop conducting when the current reaches zero — Zero Current Switching (ZCS). No forced commutation, no reverse recovery current spike. This is why the LLC achieves high efficiency with ordinary fast-recovery diodes, and even higher efficiency with SiC Schottky diodes (which have essentially no reverse recovery anyway).

This is another massive advantage over hard-switched converters and even the DAB — the secondary rectification is inherently soft-switched just because of the sinusoidal current shape.

### 6.5 The Role of L_m in the LLC

L_m serves three purposes simultaneously:

- It is the transformer magnetising inductance — it stores and releases the magnetic energy needed to magnetise the transformer core

- It participates in the lower resonance f_r2 at light/no load (when the reflected load resistance is very high and L_m is effectively in series with L_r)

- It provides the guaranteed ZVS current — at any load, including zero load, the magnetising current I_Lm flows and ensures the C_oss swing happens during dead time

This third point is critical and is what makes LLC superior to SRC (series resonant converter) for wide load range ZVS.

## 7. The CLLC Converter — Bidirectional LLC

### 7.1 Why Not Just Reverse the LLC?

The standard LLC uses a diode rectifier on the secondary. Diodes cannot conduct in reverse — you cannot push power backwards through them. To make an LLC bidirectional, you must:

- Replace the diode bridge with an active MOSFET bridge (synchronous rectification)

- Add a resonant capacitor C_r2 and series inductance L_r2 on the secondary side

This creates the CLLC topology.

### 7.2 The Symmetric Tank

[Half-Bridge 1] --- L_r1 --- C_r1 ---+--- [Transformer (L_m)] ---+--- C_r2 --- L_r2 --- [Half-Bridge 2]

|

(symmetric tank, both sides)

The CLLC has four resonant elements on the primary side (L_r1, C_r1, L_m) and a symmetric pair on the secondary (L_r2, C_r2). For symmetric bidirectional operation at unity turns ratio (or properly scaled for the turns ratio), you want:

f_r1_primary = f_r2_secondary

1/(2π√(L_r1·C_r1)) = 1/(2π√(L_r2·C_r2))

→  L_r1·C_r1 = L_r2·C_r2

(scaled by n² for non-unity turns ratio)

If the tanks are matched, the converter behaves identically in both power flow directions. If mismatched, the gain curves differ forward vs reverse — which complicates control significantly.

### 7.3 ZVS on Both Sides

In the CLLC, all four MOSFETs on both bridges must achieve ZVS. This means both bridges must have sufficient lagging current at their respective switching instants. The magnetising inductance L_m now provides the ZVS current guarantee for both sides — forward and reverse.

The dead time calculation applies to both half-bridges independently. Typically the same dead time value is used on both sides, but in practice slight asymmetry from parasitic differences between primary and secondary MOSFETs sometimes requires independent adjustment.

### 7.4 CLLC vs LLC Summary

| Feature | LLC | CLLC |
| --- | --- | --- |
| Power flow | Unidirectional | Bidirectional |
| Secondary switches | Diodes | MOSFETs |
| Tank elements | L_r, C_r, L_m | L_r1, C_r1, L_m, L_r2, C_r2 |
| ZVS coverage | Primary only | Both bridges |
| ZCS on rectifier | Yes (diodes) | Yes (MOSFETs with sync rect) |
| Typical applications | Server PSU, laptop adapters | EV OBC, battery storage, V2G |

## 8. L and C Sizing — Systematic Approach

Your friend is right that people simulate with "random" starting values. But simulation without a starting point is iterating blind. The systematic approach gives physically-motivated first estimates that will be close to optimal — then simulation fine-tunes them.

### 8.1 The Four Design Knobs for LLC

You have four free parameters: f_r, k, Q, and n. Once you fix these, all component values follow.

Step 1: Choose the turns ratio n

Set the turns ratio so that at the nominal input voltage you operate at (or very near) f_sw = f_r1 with voltage gain M = 1:

For a half-bridge LLC:

n = n₁/n₂ = V_in_nominal / (2 · V_out)

Example: 400V input, 48V output → n = 400/(2×48) = 4.17:1

This ensures your most efficient operating point is at resonance.

Step 2: Choose the resonant frequency f_r

This is primarily a size vs. switching loss trade-off:

- Higher f_r → smaller L_r and C_r → smaller magnetics and capacitors

- Higher f_r → more gate drive loss, more dead time sensitivity, tighter timing requirements

- Typical range: 100 kHz – 500 kHz for LLC in power supplies

Step 3: Choose k = L_m/L_r

k controls the gain range — how much the gain changes with frequency:

- Smaller k (3–5) → wider gain range → more frequency variation needed to regulate, but wider ZVS load range and more L_m ZVS current

- Larger k (6–10) → narrower gain range → easier frequency control, but less ZVS margin at light load

Rule of thumb: k = 4 to 7 for most designs. Start with k = 5.

Step 4: Choose Q at full load

Q determines current stress and gain curve shape:

- Higher Q → higher peak current → better filtering of harmonics → steeper gain curves (easier to regulate)

- Lower Q → lower current stress → flatter gain curves (harder to regulate at light loads)

Typical: Q = 0.3 to 0.5 at full load

Step 5: Calculate the component values

R_ac = (8·n²/π²) · R_load     where R_load = V_out²/P_out

L_r = Q · R_ac / (2π · f_r)

C_r = 1 / (Q · R_ac · 2π · f_r)

L_m = k · L_r

Verify: Z₀ = √(L_r/C_r) = Q · R_ac  ← characteristic impedance must match

The physical intuition for Z₀:
The characteristic impedance Z₀ is the tank impedance at resonance — it must match the reflected load resistance scaled by Q. A higher Q means the tank impedance is a larger fraction of the load — more reactive current circulates relative to real power. This is the same Q-factor intuition from any RLC circuit.

### 8.2 Gain Range Verification

With your chosen k and Q, verify that the gain curve covers your full input voltage range:

If V_in varies from V_in_min to V_in_max:

Required gain range: M_max/M_min = V_in_max/V_in_min

The LLC gain at f_sw → ∞ approaches: M_min = k/(k+1)  [minimum achievable gain above f_r1]

The LLC gain at f_sw = f_r2 is: M_max = (k+1)/k · 1/√(...) [from FHA formula]

If your V_in range is very wide (e.g. PFC output 350V–420V), a modest k = 5 is usually sufficient. For very wide input ranges (e.g. 200V–420V), you need a smaller k — which means larger L_m variations in your frequency range.

### 8.3 Dead-Time → L_m Constraint (Connecting Sections 4 and 8)

This is the most important sizing interaction. From Section 4.3:

I_Lm_peak ≥ C_oss_eff · V_bus / t_dead

Since I_Lm_peak = V_in / (4 · f_sw · L_m):

L_m ≤ (V_in · t_dead) / (4 · f_sw · C_oss_eff · V_bus)

This gives the maximum allowed L_m. Your calculated L_m from k·L_r must satisfy this constraint. If k·L_r is too large, you have two options:

- Reduce k (smaller L_m, but narrower gain range)

- Increase dead time (longer body diode conduction, more loss)

## 9. Dead Time — Worked Example

Let us do a complete calculation.

Design parameters:

V_bus = 400V

f_sw  = 150 kHz

L_r   = 20 μH   (from sizing calculation)

C_r   = 56 nF

L_m   = 100 μH  (k = 5)

MOSFET: GaN device, Q_oss(400V) = 90 nC (from datasheet)

Two switches in bridge → Q_oss_eff = 2 × 90 = 180 nC

Tank current at switching instant I_sw = 1.5 A (from simulation at worst-case condition)

Minimum dead time:

t_dead_min = Q_oss_eff / I_sw = 180 nC / 1.5 A = 120 ns

Target dead time (1.5× margin):

t_dead_target = 1.5 × 120 = 180 ns

Check L_m constraint:

I_Lm_peak = V_in / (4 · f_sw · L_m)

= 400 / (4 × 150×10³ × 100×10⁻⁶)

= 400 / 60 = 6.67 A

Energy in L_m: ½ × 100μH × 6.67² = 2.22 μJ

Energy in Coss: ½ × (180nC/400V) × 400² = ½ × 450pF × 160000 = 36 μJ

Wait — let's redo this properly.

Energy in C_oss_eff at 400V:

E_Coss = Q_oss_eff × V_bus / 2 = 180nC × 400/2 = 36 μJ

(note: use this form when Q_oss comes from the charge curve, not C×V²/2)

Energy in L_m at I_Lm_peak:

E_Lm = ½ × L_m × I_Lm_peak² = ½ × 100μH × (6.67)² = 22.2 μJ

E_Lm < E_Coss → ZVS is NOT achieved by L_m current alone at this dead time!

What to do: Either increase dead time, reduce L_m, or accept that at light load you rely on series tank current too. Let's reduce L_m to 60 μH (k = 3):

I_Lm_peak = 400 / (4 × 150k × 60μH) = 400/36 = 11.1 A

E_Lm = ½ × 60μH × 11.1² = 3.7 μJ  ... still less than 36 μJ

Hmm — the GaN device at 400V bus is challenging. This is why high-voltage GaN LLC designs use carefully optimised dead times and sometimes sacrifice some light-load ZVS, relying on the series tank current to provide ZVS above minimum load.

This is exactly the "calculate and hope" situation your friend described — except now you understand what you are checking and why it might not work at light load. You are not hoping randomly; you are evaluating a specific energy balance that has a clear pass/fail criterion.

What the scope should show for ZVS:

- Probe V_DS of the low-side MOSFET

- Probe V_GS of the same MOSFET

- ZVS: V_DS should reach 0V (clamped flat by body diode) before V_GS rises

- The flat section at 0V is the body diode conducting window

- If V_DS is still positive when V_GS rises → hard switching → increase dead time or reduce L_m

## 10. Summary: The Conceptual Thread

Here is the complete mental chain that connects everything:

- LC tank → sinusoidal current. The second-order differential equation of L and C has only one solution form: sinusoidal. This is physics, not design choice.

- Driving frequency vs resonant frequency → phase. When f_sw > f₀, the net reactance is inductive, current lags voltage. This is the inductive region.

- Lagging current → body diode conducting at switch transition. The switch node is already clamped to the rail before the gate arrives. This is ZVS.

- ZVS has a charge budget. The lagging current must deliver Q = C_oss_eff × V_bus during the dead time. This gives the minimum dead time formula.

- DAB: only L, no C → trapezoidal current. No second-order equation, no resonance, no sinusoidal waveshape. ZVS depends on having enough inductor current at every switching instant, which is load-dependent.

- LLC: L_r, C_r, L_m → sinusoidal current + guaranteed ZVS via L_m. Three-element tank, two resonant frequencies, frequency-controlled gain, ZCS on secondary diodes, light-load ZVS via magnetising current.

- CLLC: LLC made symmetric. Active MOSFETs on secondary, matched tanks on both sides, ZVS on all four switches.

- L/C sizing is systematic. Choose f_r, k, Q, n → calculate L_r, C_r, L_m → verify gain range → verify L_m satisfies C_oss ZVS energy budget.

- Dead time is a hard constraint, not a guess. t_dead_min = Q_oss/I_sw. L_m has a maximum value set by this constraint. Simulation verifies; the scope confirms.